SUPPORTING INFORMATION

Organophosphate esters additives and microplastics in benthic compartments from the Loire estuary (French Atlantic coast)

Castro-Jiménez, J.a,*, Aminot, Y.a, Bely, N.a, Pollono, C.a, Idjaton, B.I.Ta,1,2, Bizzozero, L.b, Pierre-Duplessix, O.b, Phuong, N.N.c, Gasperi, J.c

a IFREMER, CCEM Contamination Chimique des Écosystèmes Marins, F-44000 Nantes, France

b IFREMER, LITTORAL, F-44300 Nantes, France

c Univ Gustave Eiffel, GERS-LEE, F-44344 Bouguenais, France

*Corresponding author. Phone: +33(0)240374185;

E-mail: Javier.Castro.Jimenez@ifremer.fr

1,2 present address: (1) BRGM (Direction Eau Environnement Procédés et Analyses); 3 av. Claude-Guillemin - BP 36009, 45060 Orléans, France; (2) Université Paris Cité, Institut de physique du globe de Paris, CNRS, F-75005, Paris, France

Number of pages: 22

Number of figures: 10

Number of tables: 12

List of contents

Figure S1. Details of sampling sites in each zone

Table S1. Sample coordinates

Table S2. OPE ID and Quantification details

Text S1. Method validation

Table S3. Intermediate precision, bias and expanded uncertainty

Figure S2. Method recoveries

Table S4. Blank values in sediment samples

Table S5. Blank values in S. plana samples

Table S6. Limit of quantification (LOQs)

Table S7. MP concentrations in all sediment samples

Figure S3. Seasonal variations of MP concentrations

Figure S4. Sediment particle-size distribution

Figure S5. MP size-distribution and polymer type

Figure S6. DFs tri-OPEs and di-OPEs in sediments

Table S8. Tri-OPE concentrations in all sediment samples

Table S9. Di-OPE concentrations in all sediment samples

Figure S7. Seasonality of individual OPEs

Figure S8. Spatial variation of individual OPEs

Table S10. Wilcoxon results

Table S11. Correlations between OPEs and MPs

Figure S9. DFs tri-OPEs in S.Plana

Figure S10. OPE pattern in S. Plana

Table S12. OPE concentrations S.Plana

References

Figure S1. Details of sampling sites (blue grid) in each zone. Exact locations are provided in Table S1.

Table S1. Sample coordinates and of total organic carbon (TOC) and lipids contents (% dw)

Table S2. OPE ID and Quantification details (target compounds, surrogates, injection standards, MRM transition)

| Tri-OPEs | CAS | Cone 1 (V) | Transition 1 | Collision energy-1 (eV) | Cone 2 (V) | Transition 2 | Collision energy-2 (eV) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TEP | 78-40-0 | 10 | 183>98.9 | 16 | 10 | 183>127 | 10 |
| TEP D15 |  | 10 | 198.1>101.9 | 16 |  |  |  |
| TPrP | 513-08-6 | 10 | 225.1>98.9 | 16 | 10 | 225.1>183 | 7 |
| TPrP D21HHhhhh |  | 10 | 246.3>101.9 | 16 |  |  |  |
| TnBP | 126-73-8 | 10 | 267.1>155 | 10 | 10 | 267.1>211 | 8 |
| TnBP D27 |  | 10 | 294.4>101.9 | 18 |  |  |  |
| TiBP | 126-71-6 | 10 | 267.1>155 | 10 | 10 | 267.1>211 | 8 |
| TPeP | 2528-38-3 | 10 | 309.2>98.9 | 18 | 10 | 309.2>169.1 | 10 |
| TnHP | 2528-39-4 | 10 | 351.3>98.8 | 18 | 10 | 351.3>183 | 10 |
| TBOEP | 78-51-3 | 10 | 399.3>199 | 15 | 10 | 399.3>101 | 18 |
| TBOEP 13C2 |  | 10 | 405.3>201 | 15 |  |  |  |
| TBOEP D27 |  | 10 | 426.4>208 | 18 |  |  |  |
| TEHP | 78-42-2 | 10 | 435.4>98.9 | 16 | 10 | 435.4>113 | 12 |
| TEHP D51 |  | 10 | 426.4>102 | 16 |  |  |  |
| TCEP | 115-96-8 | 45 | 285>62.9 | 24 | 45 | 285>98.9 | 24 |
| TCEP D12 |  | 10 | 297>66.9 | 24 |  |  |  |
| TCIPP | 13674-84-5 | 10 | 327>98.9 | 22 | 10 | 327>174.9 | 12 |
| TCIPP D18 |  | 10 | 347>102 | 22 |  |  |  |
| TDCIPP | 13674-87-8 | 50 | 430.9>98.9 | 24 | 50 | 430.9>208.9 | 14 |
| TDCIPP D15 |  | 10 | 446>101.9 | 24 |  |  |  |
| TDBPP | 126-72-7 | 60 | 698.6>98.9 | 30 | 60 | 698.6>298.8 | 16 |
| TDBPP D15 |  | 60 | 713.7>102 | 30 |  |  |  |
| TPhP | 115-86-6 | 60 | 327>77 | 50 | 60 | 327>152 | 34 |
| TPhP 13C8 |  | 10 | 345.2>83 | 45 |  |  |  |
| TPhP D15 |  | 10 | 342.2>82 | 45 |  |  |  |
| TMPP | 1330-78-5 | 70 | 369.1>91 | 38 | 70 | 369.1>166.1 | 28 |
| TMPP D21 |  | 10 | 390.2>98 | 38 |  |  |  |
| IDDPP | 29761-21-5 | 40 | 391.2>251 | 10 | 40 | 391.2>77 | 50 |
| MPDPP | 26444-49-5 | 65 | 341.1>91 | 35 | 65 | 341.1>152 | 30 |
| EHDPP | 1241-94-7 | 65 | 251>77 | 28 | 25 | 363.1>251 | 8 |
| 3TBPDPP | 56803-37-3 | 10 | 383.2>152 | 45 | 10 | 383.2>327.1 | 20 |
| 3IPPDPP | 69515-46-4 | 10 | 369.1>152 | 42 | 10 | 369.1>327 | 20 |

Di-OPEs

| DnBP | 107-66-4 | 10 | 209>78.8 | 22 | 10 | 209>153 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DiBP | 6303-30-6 | 10 | 209>78.8 | 22 | 10 | 209>153 | 14 |
| DnBP D18 |  | 10 | 227.1>163 | 15 |  |  |  |
| DiBP D14 |  | 10 | 223.2>161.1 | 16 |  |  |  |
| BEHP | 298-07-7 | 10 | 321.2>78.9 | 29 | 10 | 321.2>209 | 21 |
| BBOEP | 14260-97-0 | 10 | 297>78.8 | 25 | 10 | 297>197 | 17 |
| BBOEP D8 |  | 10 | 305.2>78.8 | 25 |  |  |  |
| BDCIPP | 72236-72-7 | 10 | 317>35 | 10 | 10 | 319>35 | 10 |
| BDCIPP D10 |  | 10 | 317 >35 | 10 |  |  |  |
| DPhP | 838-85-7 | 10 | 249>93 | 30 | 10 | 249>155 | 20 |
| DPhP D10 |  | 10 | 259.1>159 | 22 |  |  |  |
| DMPP | 35787-74-7 | 10 | 277>106.9 | 25 | 10 | 277>169 | 22 |
| DMPP D14 |  | 10 | 291.2>133.9 | 30 |  |  |  |

Green colour=surrogates; red colour = injection standard

Text S1. Method validation procedure

The method was validated according to the accuracy profile procedure (NFT90-210, 2009), which consists in assessing the accuracy of the calibration function, the intermediate precision and bias and comparing it to tolerance intervals. First, the acceptance criteria were defined and tolerance intervals were determined and compared to these criteria. To evaluate the calibration function, five calibration curves, each determined for six concentration levels, were constructed over five non-consecutive days. A model from each calibration was computed and used to back-calculate concentrations. For each level, a tolerance interval was set and compared to the acceptance criteria. The domain of validity of the method can then be determined.

To evaluate intermediate precision (within-lab reproducibility) and bias (difference between the measured value obtained from repeated measurements of the same sample and a reference value), a homogenized sediment sample from a low background site was enriched with mixture of OPEs at an environmental level (1 ng g-1, except for TDCIPP 5 ng g-1). The spiked sample was analyzed in duplicate over six non-consecutive days by 2 different operators. Tolerance intervals were determined and compared to the acceptance criteria. Uncertainty was calculated by accounting for both intermediate precision and bias.

Method performance

# Calibration function

Six calibration levels were prepared ranging from 0.05 pg.µL-1 to 25 pg.µL-1 (from 0.5 pg.µL-1 to 100 pg.µL-1 for TCIPP, BEHP and DMPP and from 2.5 pg.µL-1 to 100 pg.µL-1 for BBOEP and BDCIPP) and analysed over 5 different days. For each calibration curve, the daily linear model was used to back calculate the concentration of each calibration level. With deviations between the back calculated and the nominal concentrations <20% (1-19%) for each of the 5 calibration curves, the response function was considered satisfactory for all analytes.

# Intermediate precision and bias

Intermediate precision was determined through the relative standard deviations (RSD) of the measured concentrations. The RSD ranged from 7 to 19% for seventeen out of twenty-one studied OPEs, indicating a good precision of the method. Bias, indicative of the method trueness, was determined from the difference between the measured and the nominal concentrations. The absolute bias ranged between 0 and 17%. Both precision and trueness fell within the tolerance interval determined from the Horwitz model (Horwitz 1982; Thompson 2000), showing satisfactory performance for the following 17 OPEs: TEP, TPrP, TNBP, TIBP, TPEP, TBOEP, TEHP, TCEP, TCIPP, TDCIPP, TDBPP, TMPP, MPDPP, EHDPP, DNIBP, BBOEP, DMPP. With high absolute bias for BEHP and BDCIPP (respectively 47 and 29%), and RSD 20% for 3TBPDPP and 3IPPDPP (respectively 21 and 24%), their analysis was considered semi-quantitative.

# Uncertainty

The overall uncertainty for the 17 OPEs, accounting for both intermediate precision and bias was between 21 and 48%. All details regarding obtained during validation are provided in Table S3.

Table S3. Intermediate precision, bias and expanded uncertainty

| Compound | Abbreviation | Intermediate precision (n=12. RSD%) | Bias (%) | Expanded uncertainty (%) * |
| --- | --- | --- | --- | --- |
| 2-Ethylhexyl diphenyl phosphate | EHDPP | 16 | -14 | 49 |
| Tri-cresyl phosphate | TMPP | 12 | -17 | 41 |
| Cresyl diphenyl phosphate | MPDPP | 19 | -10 | 54 |
| Triethyl phosphate | TEP | 7 | 13 | 26 |
| Tripropyl phosphate | TPrP | 15 | 10 | 48 |
| Tri-n-butyl phosphate | TNBP | 11 | -3 | 32 |
| Tri-i-butyl phosphate | TIBP | 17 | 2 | 41 |
| Tripentyl phosphate | TPEP | 10 | 1 | 30 |
| Tris(2-butoxyethyl) phosphate | TBOEP | 8 | 0 | 23 |
| Tris(2-ethylhexyl) phosphate | TEHP | 11 | -14 | 36 |
| Tris(2-chloroethyl) phosphate | TCEP | 8 | -10 | 28 |
| Tris(chloro-2-propyl) phosphate | TCIPP | 14 | 0 | 41 |
| Tris(1.3-dichloro-2-propyl) phosphate | TDCIPP | 13 | -12 | 40 |
| Tris(2.3-dibromopropyl) phosphate | TDBPP | 7 | -11 | 26 |
| Di-butyl phosphate | DNIBP | 8 | -2 | 26 |
| Bis(2-butoxyethyl) phosphate | BBOEP | 12 | -7 | 35 |
| Dicresyl phosphate | DMPP | 13 | -2 | 40 |

* Coverage factor (k) = 2

Figure S2. Method recoveries for all samples (n=40 in sediment, n=12 in S. plana)

- SEDIMENTS

- S. PLANA

Table S4. Blank values of OPE in sediment samples (in pg). nd= non-detected.

Table S5. Blank values of OPEs in Scrobicularia plana (in pg).

Table S6. Limit of quantification (LOQs) (min-max values).

Table S7. MP concentrations and polymer types in all sediment samples

Figure S3. Seasonal variations of MP concentrations in each zone (left panel) and overall seasonal variation (right panel). Kruskall-wallis test’s p values.

Figure S4. Sediment particle-size distribution in the four zones studied

Figure S5. MP size-distribution and polymer type

Figure S6. Overall detection frequencies (DFs) of tri-OPEs and di-OPEs (upper panel/green bars are Cl-OPEs) and zone-specific DFs at both seasons (lower panels).

Table S8. Tri-OPE concentrations (blank-corrected) in all sediment samples (pg/g dw)

Table S9. Di-OPE concentrations (blank-corrected) in all sediment samples (pg/g dw)

Figure S7. Overall (all zones together) seasonality of individual OPEs. Wilcoxon test p-values (n=20). Significant difference if p0.05. S=Summer; W=winter. Some box plots exclude outside values for a better visualization but all values are considered for the statistical data treatment.

Figure S8. Spatial variation of individual OPEs. Kruskal–Wallis test p-values (n=10). Significant difference if p0.05. Some box plots exclude outside values for a better visualization but all values are considered for the statistical data treatment.

Table S10. Contaminants showing significant differences among regions (Wilcoxon test)

Table S11. Correlations between OPEs and MPs (regression analysis)

Figure S9. Overall DFs of tri-OPEs in S. Plana (green bars are Cl-OPEs)

Figure S10. OPE pattern in S. Plana

Table S12. OPE concentrations in all samples from S. Plana. Values are in ng/g dry weight (dw) and lipid weight (lw).

References

Horwitz William (1982) Evaluation of analytical methods used for regulation of foods and drugs. Anal Chem 54:67–76. https://doi.org/10.1021/ac00238a002

Thompson M (2000) Recent trends in inter-laboratory precision at ppb and sub-ppb concentrations in relation to fitness for purpose criteria in proficiency testing. Analyst 125:385–386. https://doi.org/10.1039/b000282h